Sreeraaghav Raja
2/13/26

40990068

EE Design 1 Technical Report

Microcontroller Basics

Introduction

This module was a thorough introduction into the embedded systems / firmware design flow process with a microcontroller. A microcontroller is a device with multiple modules that connect to a microprocessor (single-chip CPU) like the RP2350.  The lab focused entirely on programming the Raspberry Pi Pico 2 Microcontroller and using it for two simple processes: blinking an LED and toggling a Piezo Buzzer.

Methods

Background Information:

This design implements the Raspberry Pi Pico 2 microcontroller alongside a LED, piezo buzzer, a potentiometer, a switch, and 2 resistors. The project is deliberately simple to flatten the curve of the embedded design process. Therefore, the number of components is much smaller than for later projects or even the final project for this course.

The most important component is the Raspberry Pi Pico 2 which is a microcontroller board that houses the RP2350 microcontroller chip. The RP2350 is a more powerful version of the RP2040 chip on the Raspberry Pi Pico that has these specifications (specs):

- Dual-Cortex M33 Processors at 150MHz

- 520 kB of on-chip SRAM

- GPIOs are 5 V-tolerant (powered) and 3.3 V-failsafe (unpowered)

- Peripherals:

  - 2 UART Channels

  - 2 SPI Channels

  - 2 I2C Channels

Ultimately, these specifications are not the most vital when programming the Pico 2 for this lab, but they are important as they provide a greater insight into the internal architecture of a complex device like the RP2350 [1].

Now, the term “microcontroller” has been used a few times up until this point without much of an explanation, so the following sentences will provide more information on that term. A microcontroller is very similar to a Central Processing Unit (CPU). The CPU is a part of a computer/system that is effectively “the ‘brain’ of a device” by performing arithmetic calculations, executing instructions, and controlling dataflow between the main device and peripherals [2]. The CPU is for a complete computing system and is a much larger scale than a smaller microcontroller. A microcontroller is essentially a CPU on an IC chip that also controls the Input/Output (IO) interface [3]. This makes a microcontroller a fundamental asset with a much smaller Non-Recurring Engineering (NRE) cost as it can be efficiently reproduced to be applied to a variety of situations. For example, the Raspberry Pi Pico 2 W is often also used for wireless capabilities, but it has the same chip as the regular Pico 2. The only difference lies in the peripherals on each board since the Pico 2 W has a powerful Infineon CYW43439 Chip that allows for BLE (Bluetooth Low Energy), Bluetooth, and Wi-Fi capabilities [4]. Therefore, microcontrollers can be flexible and powerful enough that they can work for a variety of different peripherals if they allow pin multiplexing, being able to configure a pin’s use for a different module.  These differences are emphasized by the different pinouts for Pico 2 and Pico 2 W as shown below.

Figure 1: Raspberry Pi Pico 2 Pinout

Figure 2: Raspberry Pi Pico 2 W Pinout

Apart from the microcontroller, the switch, piezo, and LED were the most important devices used for this lab. The LED is simple since it is a direct semiconductor that releases energy primarily as light which is useful for the case of the Red LED used in this lab. The switch is a device that can open/close circuit connections. The switch used was wired in active-high configuration as shown in the image below. An active-high/pullup switch configuration defaults to VCC/HIGH (the supply voltage) when the circuit is open, the switch is not pressed, and GND/LOW when the circuit is closed, the switch is pressed. This is advantageous because it can both provide a default state for a signal and save energy as power is only dissipated across a resistor when the switch is pressed as no current passes through the resistor in the default case. Additionally, switches are often made badly, so they bounce, oscillate in value before settling at a final value, so they require debouncing which can be done via several methods, but hardware interrupts are the most common. The last important component in this design is the piezo buzzer which implements the fundamental piezoelectric effect but in reverse. Instead of a device under mechanical pressure producing electrical charge, the presence of electrical charge deforms a mechanical device and causes it to produce sound [5]. This is a simple explanation of each of these devices, but their specs are not vital in understanding the design flow of this project.

Hardware interrupts are the most common way to debounce switches because of how accurate they can be. Interrupts are .

Figure 3: Pullup (PU) and Pulldown (PD) Switch Configurations

Design Process Methodology:

Figure 4: Altium Schematic

Here are some links on circuit block diagrams

- WIKI article on block diagrams

- Really good forum question about schematics vs block diagrams

- Free block diagram/flowchart editor: https://app.diagrams.net/

Don’t forget to write a paragraph/s that describes the figures and calls them out in the text. For example, “The code block diagram for the system using polling is shown in Figure 3. In this method, I…. (give the details)”.  Feel free to write in an active voice using first person.

Results

The results should be displayed in this section, which include figures and descriptive text for each figure. Figure captions need to be given as well, occurring below the figure. Use subheadings as necessary.

Figures needed:

- Breadboard Circuit Results:

  - Figure of waveform at the LED output showing a 2 Hz square wave

  - Figure of waveform at the buzzer output showing a 2 kHz square wave

Specific questions to answer in this section:

- How did you define the frequency of the output digital signal; when does the specified frequency not give you what you expect?

- Explain the difference between polling and interrupts and examples of when to use them.

Conclusion

The conclusion should reiterate key points learned in this module.

References

Put the references in any standard citation form here. Websites are allowed. (This is your chance to become familiar with the data sheets and manuals for your microcontroller.)

Appendix

Please paste your code for the microcontroller in this section.